# USB10 Command structure

## USB10 modes

I2C mode

I2C mode will allow the USB10 to communicate with an I2C device via USB.

To write to a device use the following format

c, ADDRESS, w, BYTES_TO_WRITE ,#...

ADDRESS 		=> The I2C slaves address formatted as a number (see below)

BYTES_TO_WRITE 	=> the number of bytes you want to write to the slave formatted as a number

#...  	=> The bytes to be written.  If you supply less bytes then BYTES_TO_WRITE, then 0 will be wrote out for each missing byte

To read a device use the following format

c,ADDRESS,r,BYTES_TO_READ

ADDRESS 		=> The I2C slaves address formatted as a number

BYTES_TO_READ 	=> the number of bytes you want to read from the slave formatted as a number

Serial mode

Serial mode will allow you to connect directly with the USB10’s UART.

To enter this mode, send the following command

S

While in serial mode, if you wish to exit the mode the PC must send down the escape characters \c

If you wish to send a hex number in the data stream from the PC you can do so by using the hex number format.

Please note that the baud rate etc is set via the virtual USBs properties

Motor Talking??

m, AXIS, BYTES_TO_WRITE,#...

AXIS 		=> 	 x for the XAXIS

y for the YAXIS

z for the ZAXIS

BYTES_TO_WRITE 	=> the number of bytes you want to write formatted as a number

#...  	=> The bytes to be written.  If you supply less bytes then BYTES_TO_WRITE, then nothing else will be written out

This has not been actually tried yet

Toggle LEDs

L,LED,COMMAND

LED		=>	r for red

g for green

COMMAND	=>	o to turn the LED on

f to turn the LED off

## Number  formats

When a command expects a number that numbers can be in the following formats

Normal

0 – 255

If you write beyond this range, the device will continue to read in the numerical values and will give an improper result

Hex

\x# or \x##

# = > 0-f

## White Space

You can include whitespace between characters to improve readability.  The program sees spaces tabs and ‘,’ as whitespace.